Dataset Analysis & Selection Dataset Analysis & Selection Report

Project: Real-Time Object Detection for Autonomous Vehicles

### ​Dataset Name: KITTI

- ​​Task Type

Primarily detection and tracking, though it’s also set up for 3D detection and segmentation tasks.

.2Number of Classes

It covers 8 categories in total. The main focus is on Cars, Pedestrians, and Cyclists, while the rest include Vans, Trucks, Trams, sitting persons, and miscellaneous objects.

.3Dataset Size

For 2D and 3D detection, there are roughly 14,999 images split almost evenly (7,481 for training, 7,518 for testing). It also contains raw video sequences and over 80,000 annotated bounding boxes.

.4Annotaion Quality

High. You get incredibly precise, professional-grade labels featuring both standard 2D boxes and 3D cuboids built from LiDAR point clouds.

.5 Environmental Diversity

​Lighting: Almost entirely clear daylight.

​Weather: Sunny and clear, meaning it severely lacks adverse weather data.

​Road Types: A mix of highways, urban, and rural roads, but they are exclusively mapped around mid-size city environments in Karlsruhe, Germany.

.5Strengths

It’s the classic industry benchmark designed specifically for the ego-vehicle (driver’s seat) perspective. The inclusion of multi modal sensor data like high res stereo cameras combined with LiDAR and GPS/IMU and the highly accurate 3D boxes are absolutely essential for spatial awareness and depth estimation.

.6Limitations

It’s just too small compared to modern standards. The lack of diverse conditions (virtually no rain, fog, snow, or night drives) means a model will easily overfit to specific European road aesthetics.

.7Recommended Model Architectures

PointPillars (great for the 3D LiDAR aspect), YOLOv8, CenterPoint, and CenterTrack

​

### ​Dataset Name: COCO

​ 1. Task Type

Object detection, instance segmentation, and keypoint tracking.

​2. Number of Classes

80 different object categories. This conveniently includes crucial driving classes like people, cars, buses, trucks, motorcycles, bicycles, traffic lights, and stop signs.

​3. Dataset Size

Massive. It packs around 330,000 images (with over 200,000 actually labeled) and roughly 1.5 million individual object instances.

​4. Annotation Quality

High. The pixel-level instance segmentation masks are highly accurate, and the standard bounding boxes are very tight.

​5. Environmental Diversity

- ​Lighting: Mixed and highly diverse.

- ​Weather: Mixed and highly diverse.

- ​Road Types: Mixed (the images are not specifically categorized by road type).

​6. Strengths

The sheer scale and visual variety of these internet-sourced images make it an amazing tool for building highly robust, generalized feature extractors. Furthermore, the detailed segmentation masks force the model to learn the actual shapes of objects rather than just relying on loose rectangular boxes.

​7. Limitations

This is fundamentally not an autonomous driving dataset. You won't find radar or LiDAR data, and it completely misses the dashboard camera perspective. Objects are frequently found in random contexlike a toy car on a table or a person in a living room rather than in actual street traffic scenarios.

​8. Recommended Model Architectures

RetinaNet, Mask R-CNN, DETR (Detection Transformer), and YOLOv8

​

### ​Dataset Name: Open Images

- ​​Task Type

Standard object detection, segmentation, and visual relationship detection.

- ​Number of Classes

It details the exact boundaries of 2.8 million objects spanning 350 classes.

- ​Dataset Size

Absolutely massive, containing around 9 million images and roughly 16 million bounding boxes.

- ​Annotation Quality

Medium. It relies on a blend of machine-generated and human verified labels. Because of the sheer volume, the bounding boxes can feel noisy or inconsistent, especially when compared to COCO’s pixel perfect masks or the 3D precision found in KITTI.

- ​Environmental Diversity

​Lighting: Mixed.

​Weather: Mixed.

​Road Types: Mixed across urban and rural setups.

- ​Strengths

The scale is unprecedented, making it one of the largest annotated computer vision datasets available. It’s packed with complex scenes featuring multiple overlapping items and rare edge case objects, which is highly useful for zero-shot or few-shot learning research.

- ​Limitations

Similar to COCO, it’s an internet scraped collection that entirely lacks the required camera angles and context for autonomous driving. The enormous size makes full training computationally expensive. Plus, the mix of crowdsourced and automated labels introduces noticeable noise.

- ​Recommended Model Architectures

EfficientDet, Faster R-CNN, YOLOv8

​

​Final Recommendation

​1. The Foundation: COCO

It's the ultimate dataset for pretraining the model. With over 330,000 images and 80 categories, it offers extreme visual diversity to help the model learn what a car or a person actually looks like. It also has precise pixel level masks.

- ​The Catch: It completely lacks the "egovehicle" (dashboard camera) perspective and doesn't have LiDAR sensor data.

​2. The Driving Specialist: KITTI

This is where the actual autonomous driving context comes in. It gives us the exact vehicle mounted camera perspective and high precision 3D bounding boxes from LiDAR, which are critical for depth estimation.

- ​The Catch: It’s very small and lacks environmental diversity like rain or night time. If we train the model only on KITTI, it will fail in real world adverse conditions.

​3. The Reject: Open Images

I would completely skip this for our real-time system. Even though it has 9 million images and over 600 classes, it completely lacks the specific camera angle and context needed for autonomous driving.

- ​The Catch: It introduces way too much irrelevant data and computational overhead. The annotations can also be noisy compared to the precision of COCO or KITTI.

​The Final Strategy: Pre train the model on COCO so it learns robust visual features, then fine-tune it on KITTI to adapt it to the 3D autonomous driving perspective.

Do not use Open Images.